Lecture 10: Parametric Distribution Analysis

1. Best Distribution Fitting: Students understand that they should avoid fitting a lot of distribution hoping to find one that fits. Instead, they should pre-select the distributions to fit based on the physics of failure (manufacturing early-life, random normal-life or wear-out end-of-life). Students should be aware that the Weibull distribution is useful for early-, normal- and end-of-life failures. Students should be aware that the log-normal distribution is a good model for failures driven by diffusion equations such as corrosion and migration. Students should be aware that exponential distributions are good for random external event normal-life failures. To identify the best fit, students should know the Anderson-Darling statistic is employed, and that the lowest the value is, the better the fit.

2. Maximum Likelihood Estimation Table: Students should be aware that once a distribution has been fitted, the MLE Table provides confidence for the distribution parameters. Further statistics like Mean Time To Failure, quartiles, means and deviations are available.

3. Censored and Uncensored Data: Students recognize the simplest life data analysis is when every trial sample runs to failure and all samples can be used for analysis. Students must know this is call “uncensored” data analysis. Students recognize the case of “censored” data, when some reliability tests had to be stopped before failure. Students must know that the presence of censored data changes the maximum likelihood regression approach.

4. Repair and Industrial Life Data: Students recognize that repair and service logs are a source of field life data, but students also understand the data is insufficient (some values might be censored, some failure modes do not appear, data on last repairs and so on might be missing).