Lecture 11: SPC & Supplier Quality

1. Keeping Processes in Control: Students understand that the control plan is a list of key process output and input variables and what to do when they become out of control. Students understand mistake proofing as the most viable approach to keeping the process in control, by altering the equipment to make sure the task cannot be done wrong. Students recognize that other reliable ways of keeping the process in control include design changes and equipment changes. Students recognize that that less reliable ways include operator training or added measurements and inspections, but they are also the cheapest ones to implement.

2. Statistical Process Control (SPC): Students understand SPC is a statistical method to evaluate the stability of the process and product variables, to indicate when the process has changed and to maintain expected results despite a normal level of variation. Particularly, students recognize that SPC eliminated special causes of variation with large effects. Students understand that a process is within statistical control if the short-term distribution parameters do not significantly change over time and there are no non-random patterns variations. Students understand that upper and lower control limits are placed on measurements to ensure that production is consistent and importantly that those control limits are different than the specification limits.  Students must know that those control limits are usually positioned 3 sigmas away from the target mean and that therefore design engineers should set tolerances 4 standard deviations away.

3. X-bar and R-chart: Students recognize that X-bar and R-bar charts are useful for processes for which a continuous variable measurement is available. Students recognize that SPC charts should be used only where the output is critical to quality or when a mistake proofing solution is not feasible. The X-bar track the mean over time for a specific average measurement generated from a sample. Students must understand that the centreline of the X-bar chart is constructed using the mean of means, and that the control limits are located at some standard deviations away from the mean of means. Students understand that the R-Chart measures the variability within sample subgroups over-time and tracks the range average. Students recognize that choosing the sample size and the sampling frequency for the calculation of the chart are important parameters. Students understand the sampling frequency is based on a rational breakpoint, usually a shift, an hour and so on. Students understand that the sample size at each point must be small enough to detect a change and large enough to be representative. Students understand that each SPC chart must be linked to an Out-of-Control Action Plan (OCAP), documenting what should be done to bring the process back into control. Students are aware that there are many chart options, and they are not limited to X-bar and R-charts, although they are very common for SPC.

4. Sample Plans: Students are aware that 100% inspection on incoming supplier parts is expensive and can be unnecessary, particularly if the parts are not safety critical. Students understand that 100% inspection is replaced with sample-based inspection, particularly when testing is destructive, has serious product risks, cost savings are expected or if measurement errors cause a lot of good parts rejections. Students recognize that sample-based inspection is primarily used for lot sentencing. Students understand the operating characteristic curve as the probability of accepting a lot based on the incoming defective rate. Students understand that the sample size (number of parts pulled from a lot) and the acceptance number (number of defectives allowed in a sample of n units) are critical sample plan variables but are aware that the only way to obtain an ideal operating characteristic curve is to perform 100% inspection. Students recognize that this ideal operating curve can be approximated with a large enough sample size. Students understand that the acceptable quality level (AQL) and lot tolerance percent defective (LTPD) define the sampling plan. Students define the AQL as the scrap rate below which the lot will be accepted and the LTPD as the scrap rate above which the lot is rejected, while in-between the AQL and LTPD, the lot might be accepted or rejected. They are defined by the alpha (producer risk) and beta (consumer risk). AQL is the highest scrap rate considered acceptable with confidence 1-alpha, and the LTPD is the lowest scrap rate above which scrapping is considered with power 1-beta.

5. Single Sampling Plan Design: Students understand a sampling plan is described by the sample size n and the acceptance number c. Students understand that instead of calculating for n and c, they can use a binomial nomograph chart, on which they can read the values for n and c given AQL, LTPD, alpha and beta. Students are aware that the binomial nomograph chart is based on the binomial equation.

6. Cpk Inspection Plans: Students understand that OC curve inspection plans are pass-fail based and that a better approach would be to sentence the lot based on whether the lot Cpk is adequate. Students understand an inspection plan can be designed around an acceptable Cpk value, based on the maximum rejectable Cpk value with alpha risk, and the minimum acceptable Cpk value with beta risk.

7. Production Part Assurance Process: Students are aware that the Production Part Assurance Process (PPAP) is a specification defining requirements that a supplier must demonstrate to become quality certified. Students understand that this document contains most points covered in the subject particularly Design FMEA, Process Flow, Process FMEA, Process Control Plan, Process Readiness Study, Initial Process Capability Studies, Measurement Systems Analysis, Dimensional Report and Production Verification Testing.