Lecture 8: Design of Experiments

1. The Experimentation Method: Understand that experimentation occurs after a list of variables has been identified and aims to determine which predictors influence the response most. Recognise that changing one or two factors is manageable but changing more than three becomes complex. Use graphs of the changed predictor against the observed variable to evaluate how sensitive the output is to the predictor. Recognise that many engineers try one-factor-at-a-time experimentation. Use design of experiments when several predictors need to be changed to avoid failing to explore the full space, particularly corner points.

2. Main Effects and Interactions: Understand full factorial experimentation as exploring all permutations of factors to cover the whole experimental space. For a full factorial analysis, list the variables to be investigated. For a full factorial analysis, define the levels to consider for each variable. For a full factorial analysis, enumerate all permutations of factor levels into a list. Evaluate the response at all permutations in the full factorial experiment. Analyse each factor for its effect on the response. Understand the main effect of a factor as the average change in the output variable produced by a change in the levels of that factor. Understand an interaction as the effect of a combined change in factors. Graph main effects and interactions to assess their magnitude. Recognise impactful main effects as large changes in the response on main-effect graphs. Recognise parallel lines as indicating absence of interaction and non-parallel lines as indicating strong interaction.

3. Design of Experiments: Understand a factor as a variable whose influence on a response is under investigation. Understand a level as one selected value of a factor. Understand a treatment combination as a combination of factor levels. Understand a run as an individual measurement of the response, similar to a row in the experimental matrix. Understand a block as a set of runs done back-to-back. Understand the formal plan for conducting experiments as a design matrix including choices of responses, factors, levels, treatments, and blocks. Understand a repeat as a repeated measurement of a treatment combination that provides a measurement of experimental error variation. Understand a replicate as a reproduced measurement of a treatment combination that informs on experimental setup error.

4. Full Factorial Models: Understand that full factorial experimentation explores all permutations of factors. Recognise that full factorial designs become inefficient as the number of predictors increases. Understand that a reduced set of experiments must still cover the experimental space. Recognise coverage properties of reduced designs as orthogonal and balanced. Recognise predetermined reduced designs including two-factorial and N-factorial designs.

5. Yates Labelling: Understand the concepts of Yates labelling. Relabel factors with alphabetical letters under Yates labelling. Relabel interactions using the corresponding factor letters. Understand that full matrices are separated into blocks which confound the generator of the block. Recognise that for two-level factors, each block has half as many experiments.

6. Partial Factorial Design: Understand that partial factorial designs can cause confounding, where different factors or interactions share the same level across multiple runs while other factors change levels. Recognise confounding in a partial factorial design as also being called aliasing. Select a block so that the confounded interaction is an irrelevant one. Understand design resolution as the level of interactions a design matrix can fit without confounding. Recognise that in resolution 3 designs, no main effects are confounded with one another, but main effects are confounded with two-factor interactions. Recognise that in resolution 4 designs, main effects are not confounded with each other or with two-factor interactions, but are confounded with three-factor interactions, while two-factor interactions are confounded with each other. Recognise that in resolution 5 designs, no main effect or two-factor interaction is confounded with another; main effects are confounded with four-factor interactions and two-factor interactions with three-factor interactions.

7. Centre Points: Understand that factorial experiments measure the response at the corners of the factor space. Recognise that factorial experiments may treat the centre as the average of measured corner responses. Recognise that the true centre response may offset quadratically from the corner-based average. Detect quadratic curvature by taking a measurement at a centre point.

8. Central Composite Designs: Understand a central composite design as an augmentation of a two-factorial design to allow quadratic and interaction terms in the response equation. Recognise that a central composite design combines a factorial design with axial points.

9. Computer Experiments and Latin Hypercubes: Understand a computer-experiment approach in which an interval is defined for each variable, points are randomly sampled within the intervals, and the response is evaluated. Recognise that random sampling within preset intervals may not yield the most efficient layout of points. Understand Latin hypercubes as providing an efficient layout of points. Recognise Latin hypercubes as equivalent to a rotated design of experiments.