Lecture 9: Reliability and Maintenance

1. Reliability: Understand reliability as the probability that a product will operate within specification for a specified duration under the designed operating conditions without failure. Understand that probability-of-failure and reliability of a component are distribution functions of time. Include the utilisation rate of a part in reliability analysis. Understand rated life in years as the ratio between the number of actuations and the usage rate in actuations per year. Recognise key reliability metrics including probability density, cumulative distribution, reliability, and hazard functions. Understand the life probability density function as the percent failures divided by the bin width in time: PDF = (# failed in interval i / Total # * time interval). Understand cumulative failed as the sum of units that have failed by a certain time. Understand the cumulative failure distribution or cumulative distribution function as the fraction of the population that has failed by a certain time: cumulative failure rate = # failed up to period i / total. Understand the survival or reliability distribution as the complement of the cumulative failure distribution, with R = 1 - CDF for a single bin i. Understand the failure rate fraction as the fraction of units that fail among those that survived up to a specific time period: r = failed in this bin / survived up until previous bin. Understand the hazard function as a rate h = pdf(current bin) / R(previous bin). Use histograms to visualise PDF, CDF, R, and hazard functions as part of non-parametric reliability analysis.

2. Reliability functions interpretations: Interpret the probability density function as providing the distribution of life for a component. Interpret the cumulative density function as the probability that a component will have failed by time t. Interpret the reliability function as the fraction of units surviving at any time and the probability that a component will survive until time t. Interpret the hazard function as the fraction dying at any time among those that have survived to that point. Associate decreasing hazard functions with early-life failures. Associate constant hazard functions with normal-life failures. Associate increasing hazard functions with end-of-life failures.

3. Distribution fitting & Weibull distribution: Recognise that parametric distributions can sometimes model reliability data more efficiently. Recognise the Weibull distribution as a useful distribution for reliability analysis. Understand the Weibull parameters beta as the shape parameter and eta as the scale parameter. Recognise beta < 1 as indicating early-life failures. Recognise beta = 1 as indicating normal-life failures. Recognise beta > 1 as indicating end-of-life failures.

4. Reliability of Systems: Understand that a system of many parts exhibits a bathtub curve for the hazard function. Fit distributions for each section of the bathtub curve when performing system reliability analysis. Recognise that system reliability often uses an exponential distribution with one parameter, the average failure rate lambda. Use the source relationships lambda = -1/MTTF and MTTF = -1/lambda. Understand that system reliability is linked to the warranty, the period during which the system will be defect free.

5. Reliability Block Diagrams: Represent the reliability of a system of many parts using reliability block diagrams. Recognise blocks in series as a scenario where the system fails if either element fails. Understand that the reliability of a system in series is the multiplication of all component reliabilities. Recognise that a series system reliability is always lower than that of the lowest-reliability element. Recognise blocks in parallel as a scenario where the system fails if both elements fail, representing redundancy in reliability design. Understand that for redundant systems, system availability can be quantified as the reliability of a system using the redundant options. Use the redundant-system reliability expression 1 - product(1 - R_i).

6. MTTF, MTBF, MTTR and hazard rates: Recognise MTTF as a natural measure for a population of parts. Recognise MTBF as a natural measure for parts that are replaced. Understand MTTR as the time measuring how long it takes to repair a failure. Understand the full time to the next failure as the sum of MTBF and MTTR. Understand the average hazard rate as the inverse of MTBF. Understand the average hazard rate for a system as the sum of the average hazard rates of its elements.

7. Maintenance plans: Understand total cost of ownership as the sum of purchase costs, operating costs, downtime costs, service costs, useful-life costs, and residual-value costs. Use the in-service rate MTBF / (MTBF + MTTR). Use the downtime rate MTTR / (MTBF + MTTR). Understand service parts cost rate as replacement-part cost divided by MTBF + MTTR. For a long-time interval T, express replacement-part cost over time by multiplying the replacement-part cost by T as described in the source. Recognise that multiple service strategies can be viable, particularly preventive maintenance for wear-out failures. Recognise end-of-life pathways such as repairs, recycling, and landfill.